# Assignment 3

Student Name: Priyanshu

Enrollment/Roll Number: IoT-029

Course Name:  Internet of Things (IoT)

Assignment Title:

Submission Date:

Ans35.

WeatherFetcher.ino

Config.h

Output

Ans36.

| Feature | MQTT | HTTP |
| --- | --- | --- |
| Architecture Pattern | Publish/Subscribe using a broker | Client/Server (Request-Response) |
| Data Transfer Model | Sends data only when required through topics | Client sends a request and waits for a response |
| Power Consumption | Low power consumption | Higher power consumption |
| Latency | Very low latency | Higher latency compared to MQTT |
| Use Case Suitability for IoT | Excellent for IoT devices, sensors, and real-time monitoring | Better for web applications, APIs, and occasional data exchange |
| Security Considerations | Supports TLS/SSL encryption, authentication, and access control through the broker | Supports HTTPS (SSL/TLS), authentication, and secure APIs |

For a 1000-node smart agriculture system, I would choose MQTT.

Reason:

- It uses very little bandwidth, making it suitable for large numbers of sensor nodes.

- It consumes less power, which helps battery-operated devices last longer.

- It provides low latency, allowing quick delivery of sensor readings and alerts.

- The publish/subscribe model makes communication efficient because devices only receive the data they subscribe to.

- A single MQTT broker can efficiently manage communication among thousands of devices, making the system scalable.

Ans37.

Wi-Fi security is very important in IoT because most smart devices send and receive data over the internet. If the network is not secure, attackers can steal information or even take control of the devices. One common attack is based on WPA2 vulnerabilities. Although WPA2 is a secure Wi-Fi standard, some weaknesses, such as the KRACK attack, have shown that attackers can exploit devices that have not been updated with security patches.

Another common attack is the Evil Twin attack. In this attack, a hacker creates a fake Wi-Fi network with the same name as a real one. If an IoT device connects to this fake network, the attacker may be able to capture data or steal login credentials.

To make ESP32 devices more secure, developers should use WPA3 whenever possible because it provides better protection than WPA2. It is also a good idea to use TLS encryption with certificate pinning, so the ESP32 communicates only with trusted servers. Keeping the firmware updated, using strong passwords, and disabling unnecessary services also improve security.

Hardcoding Wi-Fi passwords or API keys directly into the source code is risky. If someone gets access to the code or uploads it to a public GitHub repository, anyone can see those credentials and misuse them. A better approach is to store passwords in a separate configuration file or use secure storage, and never upload sensitive information to public repositories.

Ans38.

LPWAN stands for Low Power Wide Area Network. It is a type of wireless network designed for IoT devices that need to send small amounts of data over long distances while using very little battery power. LPWAN is commonly used in smart agriculture, smart cities, environmental monitoring, and asset tracking because devices can operate for several years on a single battery.

LoRa is one of the most popular LPWAN technologies. It achieves long-range communication by using a technique called the Spreading Factor (SF). A higher spreading factor allows the signal to travel farther and improves communication in areas with weak signal strength. Although increasing the spreading factor reduces the data transmission speed, it helps maintain reliable communication over long distances while keeping power consumption low.

When compared with other wireless technologies, LoRa provides the longest communication range, often several kilometers, and is suitable for outdoor IoT applications. Zigbee has a shorter range but supports mesh networking, making it useful for home automation and industrial control. Z-Wave is mainly used in smart home devices and also has a shorter range than LoRa.

A real-world example in India is the use of LoRa-based smart agriculture solutions in states such as Maharashtra and Karnataka. Farmers use LoRa sensors to monitor soil moisture, temperature, and water levels in their fields. The collected data helps them make better irrigation decisions, reduce water wastage, and improve crop productivity.

Ans39.

An IoT gateway is a device that acts as a bridge between IoT sensor nodes and the cloud. It collects data from sensors, processes or filters it if needed, and then sends it to the cloud. It can also receive commands from the cloud and forward them to the connected IoT devices. An IoT gateway improves security, reduces network traffic, and allows devices using different communication protocols to work together.

Diagram

Protocols Used at Each Layer

| Layer | Common Protocols |
| --- | --- |
| IoT Sensor Nodes → Gateway | LoRa, Zigbee, Bluetooth, Wi-Fi |
| Gateway → Cloud | MQTT, HTTP/HTTPS, CoAP |
| Cloud → User Application | HTTPS, REST API, WebSocket |

Ans40.

QoS (Quality of Service) in MQTT defines how reliably a message is delivered between the publisher and the subscriber. MQTT provides three QoS levels.

QoS 0 – At Most Once
The message is sent only once without any confirmation from the receiver. If the message is lost, it is not sent again. This is the fastest method and uses the least bandwidth, but delivery is not guaranteed.

QoS 1 – At Least Once
The sender waits for an acknowledgment from the receiver. If no acknowledgment is received, the message is sent again. This guarantees that the message reaches the receiver, but it may be delivered more than once.

QoS 2 – Exactly Once
This is the highest level of reliability. It uses a four-step handshake to ensure that the message is delivered only once. It is slower and consumes more bandwidth but prevents duplicate messages.

For an IoT water level monitoring system, I would choose QoS 1

Reason:
In a water level monitoring system, it is important that sensor readings reach the server reliably. If a reading is accidentally sent twice, it usually does not cause a major problem because the latest value can be processed. However, losing a water level reading could result in incorrect monitoring or delayed alerts. QoS 1 provides a good balance between reliability, speed, and network usage, making it the most suitable choice for this application.

Ans41.

Objective

The aim of this project is to design an IoT-based Smart Home Air Quality Monitoring System that continuously monitors indoor air quality using an MQ-2 gas sensor and measures temperature and humidity using a DHT11 sensor. The system alerts users through an RGB LED, buzzer, and Bluetooth notifications whenever the air quality becomes unsafe. It is also capable of working without an internet connection because Bluetooth communication is used for local alerts.

Components Used

- ESP32 Development Board

- MQ-2 Gas Sensor (Simulated using a potentiometer in Wokwi)

- DHT11 Sensor (DHT22 used in Wokwi for simulation)

- RGB LED

- Active Buzzer

- Bluetooth Serial

- Jumper Wires

- Breadboard

Working Principle

The ESP32 continuously reads the gas level from the MQ-2 sensor and the temperature and humidity from the DHT11 sensor. Based on the gas sensor reading, the system classifies the air quality into three levels: Safe, Moderate, and Danger.

If the air quality is safe, the green LED glows and no alarm is generated. When the smoke level increases, the LED changes to yellow and the buzzer gives a warning beep. If the smoke level becomes dangerous, the LED turns red and the buzzer produces a faster alarm. At the same time, warning messages are sent to a mobile phone through Bluetooth. All sensor readings are also logged on the Serial Monitor in CSV format for easy monitoring and analysis.

Threshold Design

| MQ-2 Reading | Air Quality | LED | Buzzer |
| --- | --- | --- | --- |
| 0 – 299 | Safe | Green | OFF |
| 300 – 599 | Moderate | Yellow | Slow Beep |
| 600 – 4095 | Danger | Red | Fast Beep |

Sensor Calibration

Since the MQ-2 sensor is not available in Wokwi, a potentiometer is used to simulate the gas sensor. By rotating the potentiometer, different smoke levels are generated. During testing, the analog values were divided into three ranges representing safe, moderate, and dangerous air quality. These threshold values can be adjusted according to the actual MQ-2 sensor readings in a real hardware implementation.

CSV Output Format

Timestamp, MQ-2 Value, Temperature, Humidity, Severity

00:01,220,23.4,49,SAFE
00:02,430,23.5,50,MODERATE
00:03,765,23.7,49,DANGER

Bluetooth Alert Messages

SAFE

SAFE
Gas:210
Temp:22.8°C
Humidity:51%

MODERATE

WARNING
Moderate Smoke Detected
Gas=430
Check Fireplace

DANGER

DANGER!!
Smoke Level Critical
Gas=765
Open Windows
Leave Room if Necessary

Advantages

- Monitors indoor air quality in real time.

- Provides visual, audio, and Bluetooth alerts.

- Works without an internet connection.

- Stores sensor data in CSV format for monitoring.

- Can be used in homes, offices, and small industries.

Applications

- Smart homes

- Indoor air quality monitoring

- Fire and smoke detection

- Warehouses

- Offices and commercial buildings

Ans42.

Objective

The objective of this project is to automate irrigation for a small saffron farm by monitoring soil moisture, temperature, and weather conditions. The system waters the plants only when the soil is dry and avoids watering during rainy or overcast conditions. It also allows the farmer to manually control watering through a push button while logging all watering events to Google Sheets.

Components Used

- ESP8266 (NodeMCU)

- Soil Moisture Sensor (Simulated using Potentiometer in Wokwi)

- LDR (Rain Proxy)

- Relay Module (Simulated using LED)

- Push Button

- 16×2 I2C LCD

- Wi-Fi Connection

- Google Sheets (via IFTTT/Google Apps Script)

Working Principle

The ESP8266 continuously checks the soil moisture level at regular intervals. Based on the sensor reading, the soil is classified as Dry, Optimal, or Wet.

Before turning on the water pump, the system checks two additional conditions:

- The LDR sensor detects whether the sky is overcast, which acts as a rain indicator.

- The OpenWeatherMap API checks if rain is expected or currently occurring.

The pump is activated only when the soil is dry and there is no indication of rain. A push button allows the farmer to manually start watering whenever required. After manual activation, the system enters a 5-minute lockout period to prevent repeated watering.

The current soil moisture, operating mode, and last watering time are displayed on the 16×2 LCD, while every watering event is logged to Google Sheets through Wi-Fi.

Threshold Design

| Soil Moisture | Condition | Pump Status |
| --- | --- | --- |
| 0–29% | Dry | ON (If no rain is detected) |
| 30–70% | Optimal | OFF |
| 71–100% | Wet | OFF |

Rain Detection Logic

The system uses two methods to avoid unnecessary watering:

- LDR Sensor: Detects low light or overcast conditions, which act as a rain proxy.

- Weather API: Fetches live weather information from OpenWeatherMap to check if it is raining or rain is expected.

The pump is turned OFF if either of these conditions indicates rain.

Manual Override

A push button is provided for manual watering.

- Pressing the button immediately starts the pump.

- After manual activation, the system ignores further button presses for 5 minutes.

- This lockout prevents accidental repeated watering and protects the saffron crop from overwatering.

Google Sheets Logging

Every watering event is automatically logged to Google Sheets.

| Time | Moisture | Rain | Pump | Mode |
| --- | --- | --- | --- | --- |
| 10:00 | 25% | No | ON | AUTO |
| 10:30 | 55% | No | OFF | AUTO |
| 11:00 | 24% | Yes | OFF | AUTO |

Ans43.

Objective

The objective of this project is to develop a secure IoT-based smart door lock system for a university hostel. Instead of using physical keys, students enter their 4-digit ID through a keypad. The system then generates a 6-digit One-Time Password (OTP) and sends it to the student's phone via Bluetooth. The door unlocks only if the correct OTP is entered within 30 seconds, providing an additional layer of security.

Components Used

- ESP32

- 4×4 Matrix Keypad

- 16×2 LCD (or 0.96" OLED Display)

- HC-05 Bluetooth Module (or ESP32 Built-in Bluetooth)

- Servo Motor (SG90)

- Jumper Wires

- Breadboard

Working Principle

When the system starts, the LCD prompts the user to enter a 4-digit student ID using the keypad. As each digit is entered, it is displayed as an asterisk (*) to keep the ID hidden.

After validating the ID, the ESP32 generates a random 6-digit OTP and immediately sends it to the student's phone through Bluetooth. At the same time, a 30-second countdown timer starts using the millis() function, ensuring the program remains responsive without using delay().

The student enters the received OTP using the keypad. If the entered OTP matches the generated OTP within the time limit, the servo motor rotates 90°, simulating the unlocking of the hostel door. If the OTP is incorrect or the timer expires, access is denied, and the servo remains locked.

Every login attempt, whether successful or failed, is recorded in the Serial Monitor for future monitoring.

Program Logic

Step 1 – Student ID Verification

- Student enters a 4-digit ID.

- Digits are displayed as * * * * for privacy.

Step 2 – OTP Generation

- ESP32 generates a random 6-digit OTP.

- OTP is sent to the student's phone through Bluetooth.

Step 3 – Timer Starts

- A 30-second countdown begins using millis().

- No delay() is used.

Step 4 – OTP Verification

- Student enters the OTP.

- If it matches within 30 seconds, the door unlocks.

- Otherwise, access is denied.

Step 5 – Door Control

- Servo rotates 90° for successful authentication.

- Servo returns to 0° after a few seconds.

Step 6 – Logging

Every attempt is logged to the Serial Monitor.

Ans44.

Objective

The objective of this project is to develop a low-cost IoT-based security system for a small shop. The system detects motion using a PIR sensor during configured shop closing hours. When motion is detected, it activates a buzzer with three alarm levels, flashes LEDs sequentially to simulate a security light, logs the event with timestamps on the Serial Monitor, and sends an alert to the shop owner through a Telegram bot using the ESP32 Wi-Fi module.

Components Used

- ESP32 Development Board

- PIR Motion Sensor

- Potentiometer (Sensitivity Adjustment)

- Active Buzzer

- 3 LEDs (LED Strip Simulation)

- Wi-Fi Connection

- Telegram Bot

- Jumper Wires

- Breadboard

Working Principle

The ESP32 continuously monitors the PIR sensor for motion. The shop owner can configure the monitoring period using the Serial Monitor by entering a command such as:

SET_HOURS 22 06

This configures the system to operate between 10:00 PM and 6:00 AM.

A potentiometer is used to adjust the PIR sensor sensitivity by changing the motion detection threshold. If motion is detected during the configured security period, the system immediately activates the alert sequence.

The alert begins with a Warning stage, where the buzzer emits slow beeps and LEDs flash slowly. If motion continues, the system escalates to the Alarm stage with faster beeps and quicker LED flashing. If motion still persists, it enters the Urgent stage, producing a continuous buzzer sound and rapid LED flashing.

At the same time, the ESP32 sends a notification to the owner's Telegram account and records the event along with the timestamp in the Serial Monitor.

Pin Connections

| Component | ESP32 Pin |
| --- | --- |
| PIR Sensor Output | GPIO 34 |
| Potentiometer | GPIO 35 |
| LED 1 | GPIO 18 |
| LED 2 | GPIO 19 |
| LED 3 | GPIO 21 |
| Active Buzzer | GPIO 22 |

Alert Escalation

| Motion Duration | Level | LED Pattern | Buzzer |
| --- | --- | --- | --- |
| 0–5 sec | Warning | Slow Sequential Flash | Slow Beep |
| 5–10 sec | Alarm | Medium Flash | Fast Beep |
| More than 10 sec | Urgent | Rapid Flash | Continuous Tone |